Hardware Version: V1.0

Software Version: V16.01.0.13

Upgrade Notes:

- This firmware is only fit for EW15DV1.0 and its firmware version must be in V16.01.0.XX.

- Do not power off the device when upgrading.

- Please unzip the file you downloaded and use the file ended with "bin" or "trx" to upgrade your device.

- Please upgrade the firmware for all the devices in the network and reset them to factory settings to make sure the node connect to each other because this firmware upgrade the Mesh Link connection method.

Release Notes:

- Modify the Mesh Link connection method.

- Modify the default operating mode from bridge mode to router mode to make sure the end device can get the correct IP address from the mesh router.

- Modify the SSID synchronization mechanism that all the SSID of root node can synch to satellite nodes.

- Modify the Guest SSID synchronization mechanism that all the guest SSID of root node can synch to satellite nodes.

- Fix the RF closed problem when the device get the configuration from cloud

- Fix the memory leak problem that will make the device system crash after the device running about 40 days.

- Add Mesh Link mode option to choose Kit mode and single mode, when the mesh router is one of the Kit Package, the default mesh link mode is Kit mode and the satellite node will connect to root node anytime. And if you set single mode for mesh router of the Kit package, it can connect to other mesh group.

- Add Sync with uplink node option for satellite node that enable the satellite node can configure wireless parameters.